Manual Magnaprobe RTK expansions

By: Achille Capelli and Anika Pinzer, 2024

# Table of Contents

Table of Contents	1

Download files from Magnaprobe	1

PPK of septentrio files	2

PPK of RINEX files	3

Merge files	3

# Download files from Magnaprobe

Use cable and DevConf from Campbell or USB dongle

## DevConfig

- Connect with  RS232-USB adapter

- Open DevConfig

- Select COM port and logger CR800 series 🡪 Connect

- Download data in Data collection Tab, Choose folder, select OperatorView, Start

# Septentrio files

## Download Files (.sbf)

- Download .sbf file over USB and 192.168.3.1 in browser

- Alternative take out SD card and read

- Naming convention: Folder: YYddd (eg. 24336), filename: STAT_ddd0.24_ (eg. Mg1_3360.24_)

## Extract RINEX files

- Use RxTools:SBF Converter to extract RINEX and ASCII .txt files

Opening the .sbf files

- open SBF converter app (in RX Toolbox)

- use: RINEX, the events (klicks) are saved int he ASCII format

- klick YES on window that opens

- creates a .txt file and an P and an O file

- Event A = Magnaprobe button

- Event B = Red button

## PPK of RINEX files

- With Emlid Studio postprocess (PPK) data using Reach base station data and the exported RINEX data

- RTKLIB can be used as an alternative (less user friendly)

Emlid Processing

- OPUS correct base station files (see below on how to do it)

- open Emlid Studio

- Rover/ Base: O file

- Navigation file: P

- Antenna height of sea ice rod: 211.6cm (measured from the bottom of the rod to the bottom of the GNSS antenna –

- The SeptentrioX5 were set up with the antenna information of the TOP106 antenna (white geodetic) so the position is the ARP (antenna reference point) of the antenna.

# OPUS correcting the base file

- go to: https://www.ngs.noaa.gov/OPUS/

- upload the base station .O file

- if file is too large, open Emlid Studio software using ‘Convert to RINEX’

  - open settings

  - set ‘Interval’ to 30s instead of 1s

  - convert file and use that instead for opus correction

As alternative download CORS data and use static data processing in EMLID float.

# Merge files

### Python script in Spyder

- Spyder: open manuscripts

- Run magnamerge file first then the merge file

### Command line

- Use python script mergeMagna.py

- Open command line (Anaconda PowerShell)

- Navigate to root directory (cd d:)

- Example command:

python .\scripts\mergeMagna.py   magna009.sbf_SBF_ExtEvent1.txt   SEPT2910.pos MagnaProbeRTK_2024-10-17.dat   path=.\data\241017_TestRaptor\Magnaprobe

Syntax: mergeMagna.py file_events  file_cont  file_Magna  plot=True, dt_min=0.1, dt_mean=0.5, tolerance=0.5, DeltaH=8, DeltaS=17

Parameters

----------

filename_events : TYPE

Event data GNSS.   .txt from RXTools or .pos form Emlid studio

filename_cont : TYPE

Continus data GNSS.  .pos form Emlid studio

file_magna : TYPE

File data magnaprobe.

file_save : TYPE, optional

Filename for saving data. The default is 'PPK_Magna.csv'.

file_saveB : TYPE, optional

Filename for saving events from button B. The default is 'PPK_Magna_B.csv'.

path: string, optional

path of files. The default is '\\'.

plot : float, optional

Produce plot to check sync. The default is True.

dt_min : float, optional

filter events nearer then dt_min. The default is 0.1.

window : float, optional

Window size for averaging continuous data around events in 's'. The default is 1.

tolerance : float, optional

tolerance in 's'for syncing Magnaprobe and GNSS. The default is 1.

DeltaH : float, optional

Time shift Magnaprobe in hours. GPS time to local time. The default is 8.

DeltaS : float, optional

Time shift Magnaprobe in seconds (leap seconds+ error). The default is 16.62